ATMO/ECE 489/589                                       Homework #7                                       Spring 2007

1a.	What output voltage would you observe on

the flat plate antenna system shown at right

assuming a peak dE/dt signal amplitude of

100 V/(m μs)?

1b.	Assume the dE/dt signal was produced by a

lightning discharge 40 km away.  Use the

transmission line model expression to estimate

the peak dI/dt in the discharge (assume a return

stroke velocity of 2 x 108 m/s).

2.	The graph below compares simultaneous independent measurements of peak E field and 	peak current amplitudes in 28 triggered lightning return strokes. (adapted from J.C. 	Willett, J. C. Bailey, V.P. Idone, A. Eybert-Berard, L. Barret, "Submicrosecond 	Intercomparisons of Radiation Fields and Currents in Triggered Lightning Return Strokes 	Based on the Transmission-Line Model," J. Geophys. Res., 94, 13275-13286, 1989.)

2a.	What would you expect to see if the transmission line model expression relating return 	stroke peak I and peak E radiation field amplitudes is valid?

2b. 	Verify the statement that a diagonal line from upper left to lower right corresponds to 	a velocity of 1.7 x 108 m/s?