ATMO/ECE 489/589 Spring 2011

Homework #1 due Tue., Feb. 1

1.	(8 pts) The fair-weather electric field at the earth's surface is approximately 200 V/m and points downward.  At an altitude of 1500 m, the field still points downward but is only 20 V/m.  What is the average volume charge density in the atmosphere between the ground and 1500 m?  What polarity does this space charge have?

2.	(10 pts) A lightning leader is often modeled as having a narrow diameter conducting core surrounded by a cylindrical envelope of space charge.  Assuming that the channel is infinitely long and straight, and that the total charge per unit length on the leader is ρo (C/m), derive an expression for the electric field, E(r), everywhere inside and outside the space charge radius.  You can assume that the volume charge density within R is uniform, and you can ignore the conducting core.

In practice R will be controlled by the local breakdown electric field, EB,  Estimate the minimum R when ρo = 10-3 C/m and EB = 3 x 106 V/m.

3.	(12 pts) A point charge, Q, is located at an altitude, H, above a grounded conducting plane of infinite extent.

(a) Is there an attractive or repulsive force between Q and the conducting plane?

(b) Derive an expression for the electric field at the ground as a function of horizontal distance, D (you can assume the ground is flat; it’s not drawn very well below).

(c) Determine the surface charge density as a function of D, σ(D).

(d) Integrate the surface charge density over the entire conducting plane.

4.	(EC*) Verify the following

*EC means extra credit (sort of).  Any points earned on this extra credit question can be used to make up for points missed on Questions 1 – 3.  You can earn up to a maximum of 30 pts on this assignment